To Set the Date and Month on DiscoveryCam

- With or without SD card inserted, power Unit ‘ON’ and wait for it to “Boot Up”.

- When video ‘noise’ or camera image is present, press the ◄ or ► button in the SD portion of the remote.

- “Playback Mode” appears.  Press ‘SET UP’.

- Date & Time info box appears with the DATE line highlighted in Red.

- Press ‘SAVE’ once to isolate YEAR data

- Press  ◄ to increase or ►to decrease entry

- Press ‘SAVE’ when proper data is reached.

- Field shifts to MONTH (01 thru 12).  Repeat as above to select & save.

- Field shifts to DAY (01 thru EOM).  Repeat as above to select & save.

- When all data Is entered, the entire Data field line returns to Red.

- Press the ◄ button to move to the TIME line

- Press ‘SAVE’ once to highlight the HOURS field (00 – midnight) thru 23 (11pm)

- Repeat as above to select and save

- Field shifts to to MINUTES (00 thru 59)

- Repeat as above to select and save

- End of user programming.  Press ‘SETUP’ button to exit Setup.